Types of Containers

Standard containers

Description

Standard containers are also known as general purpose containers. They are closed containers, i.e. they are closed on all sides. A distinction may be drawn between the following types of standard container:

|  | Standard containers with doors at one or both end(s) |
| --- | --- |
|  | Standard containers with doors at one or both end(s) and doors over the entire length of one or both sides |
|  | Standard containers with doors at one or both end(s) and doors on one or both sides |

In addition, the various types of standard container also differ in dimensions and weight, resulting in a wide range of standard containers.

Standard containers are mainly used as 20' and 40' containers. Containers with smaller dimensions are very seldom used. Indeed, the trend is towards even longer dimensions, e.g. 45'. 

Frame and bottom cross members are made of steel profiles, while three different materials are used for the walls:

1. Steel sheet, corrugated

Characteristics:

|  | low material costs |
| --- | --- |
|  | easy to repair |
|  | high tare weight |
|  | susceptible to corrosion |
|  | difficult to clean owing to corrugated walls |

2. Aluminum sheet in conjunction with stiffening profiles

Characteristics:

|  | low tare weight |
| --- | --- |
|  | high material costs |
|  | easily deformed, very quickly dented |

3. Plywood with glass fiber-reinforced plastic coating (plywood + GRP)

Characteristics:

|  | easy to clean owing to smooth surfaces |
| --- | --- |
|  | easy to repair |
|  | strong and resilient, does not dent |
|  | moderate material costs |
|  | moderate tare weight |

The cost advantages have led to the predominant use of steel for container walls. 

The floor is generally made of wood, usually planking or plywood. Although wood is relatively expensive, it has substantial advantages over other materials: it is strong and resilient, does not dent, may be easily replaced during repairs and, when appropriately finished, has an adequate coefficient of friction, which is important for cargo securing.

Standard containers may additionally be equipped with certain optional extras:

|  | Forklift pockets: these allow handling of empty containers with forklift trucks. Packed containers must not be picked up in this way unless specifically permitted. Forklift pockets are installed only in 20' containers and are arranged parallel to the center of the container in the bottom side rails. 40' containers do not have forklift pockets, since the pockets are relatively close together and such large containers would be difficult to balance. In addition, the forklift truck travel paths are often not wide enough. |
| --- | --- |
|  | Gooseneck tunnel: Many 40' containers have a recess in the floor at the front end which serves to center the containers on so-called gooseneck chassis. These recesses allow the containers to lie lower and therefore to be of taller construction. |
|  | Grappler pockets: In general, containers are handled by top spreaders using the corner fittings or corner castings. However, some containers have grappler pockets for handling by means of grapplers. |

Special fittings are available for transporting special cargoes:

|  | Clothes rails for hanging garments: Special lashing rings attached to the top side rail serve to accommodate clothes rails on which textiles may be transported hanging on clothes-hangers. These are often used in the East Asia import trade. Additional lashing rings are installed on the bottom side rail and the corner posts. |
| --- | --- |
|  | Inlet (bulk bag or liquid bulk bag): Plastic liners may be suspended in standard containers for transporting bulk cargo or nonhazardous liquids. |

The wooden components of most containers are impregnated against insect infestation, since, when lumber is used, it may, under certain circumstances, be necessary to comply with the quarantine regulations of the country of destination and a phytosanitary certificate may have to be enclosed with the shipping documents. Information may be obtained from the phytosanitary authorities of the countries concerned.

Dimensions/weights

The following are some of the most important details relating to standard container types. The data was taken from Hapag-Lloyd, Hamburg

| Standard container of steel: 20' long and 8'6" high with corrugated walls and wooden floor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  | Door openings |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height [mm] | Width [mm] | Height [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |  |
| 5895 | 2350 | 2392 | 2340 | 2292 | 30480 | 2250 | 28230 | 33.2 |
| 5895 | 2350 | 2385 | 2338 | 2292 | 24000 | 2250 | 21750 | 33.2 |

| Standard container of steel: 40' long and 8'6" high with corrugated walls and wooden floor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  | Door openings |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height [mm] | Width [mm] | Height [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |  |
| 12029 | 2350 | 2392 | 2340 | 2292 | 30480 | 3780 | 26700 | 67.7 |

High-cube containers

Description
High-cube containers are similar in structure to standard containers, but taller. In contrast to standard containers, which have a maximum height of 2591 mm (8'6"), high-cube containers are 2896 mm, or 9'6", tall. High-cube containers are for the most part 40' long, but are sometimes made as 45' containers. 

A number of lashing rings, capable of bearing loads of at most 1000 kg, are mounted on the front top end rail and bottom cross member and the corner posts.

Many 40' containers have a recess in the floor at the front end which serves to center the containers on so-called gooseneck chassis. These recesses allow the containers to lie lower and therefore to be of taller construction.

Dimensions/weights

The following are some of the most important details relating to high-cube container types. The data was taken from Hapag-Lloyd, Hamburg.

| High-cube container of steel: 40' long and 9'6" high with corrugated walls and wooden floor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  | Door openings |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height [mm] | Width [mm] | Height [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |  |
| 12024 | 2350 | 2697 | 2340 | 2597 | 30480 | 4020 | 26460 | 76.3 |

Hard-top containers

Description

The walls of hard-top containers are generally made of corrugated steel. The floor is made of wood.

It has two typical distinguishing structural features. On the one hand, it is equipped with a removable steel roof. In some types, this roof has points for accommodating forklift trucks, allowing the roof to be lifted by forklift truck. The roof weighs approx. 450 kg. In addition, the door header may be swivelled out. 

These two structural features greatly simplify the process of packing and unpacking the container. In particular, it is very easy to pack and unpack the container from above or through the doors by crane or crab when the roof is open and the door header is swivelled out.

In the case of transport of an overheight cargo, the container roof may be left open and fastened directly to a side wall on the inside of the container. To do this, the roof only needs approx. 13 cm (5 1/8") of space.

Lashing rings, to which the cargo may be secured, are installed in the upper and lower side rails, the corner posts and the middle of the side walls. The lashing rings on the side rails and corner posts may take loads of up to 2000 kg. The lashing rings in the middle of the side walls may take loads of up to 500 kg, provided that the roof is closed.

Usual hard-top container dimensions are 20' and 40'.

Dimensions/weights

The following are some of the most important details relating to hard-top containers. The data was taken from Hapag-Lloyd, Hamburg.

| Hard-top container of steel: 20' long and 8'6" high with corrugated walls and wooden floor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  | Door | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height (middle) [mm] | Height (side) [mm] | Max. width [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |  |
| 5886 | 2342 | 2388 | 2313 | 2336 | 30480 | 2700 | 27780 | 32.8 |
| 5886 | 2342 | 2375 | 2330 | 2336 | 30480 | 2590 | 27890 | 32.8 |

| Hard-top container of steel: 40' long and 8'6" high with corrugated walls and wooden floor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  | Door | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height (middle) [mm] | Height (side) [mm] | Max. width [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |  |
| 12020 | 2342 | 2388 | 2313 | 2336 | 30480 | 4700 | 25780 | 67.2 |
| 12020 | 2345 | 2380 | 2300 | 2334 | 30480 | 4700 | 25780 | 65.3 |

| High-cube hard-top container of steel: 40' long and 9'6" high with corrugated walls and wooden floor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  | Door | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height (middle) [mm] | Height (side) [mm] | Max. width [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |  |
| 12020 | 2342 | 2693 | 2618 | 2336 | 30480 | 4900 | 25580 | 75.8 |
| 12020 | 2342 | 2693 | 2618 | 2336 | 32500 | 5200 | 27300 | 76.0 |

Dimensions of roof and door openings

The meaning of the individual letters is clear from the following Figures:

| Figure 3 | Figure 4 |
| --- | --- |

| 20' hard-top container |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Roof openings |  | Door openings |  |  |  |  | Reduced widths when roof is carried inside |  |  |
| Length | Width | Width | Width | Width | Height | Height | Internal width [mm] | Roof opening width [mm] | Door opening width [mm] |
| B [mm] | C [mm] | F [mm] | G [mm] | H [mm] | I [mm] | K [mm] |  |  |  |
| 5590 | 2208 | 2336 | 1896 | 2208 | 2276 | 2220 | 2209 | 2142 | 2206 |
| 5590 | 2208 | 2336 | 1896 | 2208 | 2292 | 2220 | 2209 | 2142 | 2206 |
| 5590 | 2208 | 2336 | 1896 | 2208 | 2280 | 2231 | 2215 | 2148 | 2212 |

| 40' hard-top container |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Roof openings |  | Door openings |  |  |  |  | Reduced widths when roof is carried inside |  |  |
| Length | Width | Width | Width | Width | Height | Height | Internal width [mm] | Roof opening width [mm] | Door opening width [mm] |
| B [mm] | C [mm] | F [mm] | G [mm] | H [mm] | I [mm] | K [mm] |  |  |  |
| 11724 | 2208 | 2336 | 1896 | 2208 | 2292 | 2220 | 2209 | 2142 | 2206 |
| 11724 | 2208 | 2336 | 1896 | 2208 | 2276 | 2220 | 2209 | 2142 | 2206 |
| 11724 | 2208 | 2334 | 1882 | 2208 | 2290 | 2125 | 2205 | 2102 | 1996 |

| 40' high-cube hard-top container |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Roof openings |  | Door openings |  |  |  |  | Reduced widths when roof is carried inside |  |  |
| Length | Width | Width | Width | Width | Height | Height | Internal width [mm] | Roof opening width [mm] | Door opening width [mm] |
| B [mm] | C [mm] | F [mm] | G [mm] | H [mm] | I [mm] | K [mm] |  |  |  |
| 11724 | 2208 | 2336 | 1896 | 2208 | 2597 | 2525 | 2230 | 2163 | 2227 |

Open-top containers

Description

The walls of open-top containers are generally made of corrugated steel. The floor is made of wood.

It has the following typical distinguishing structural features. The roof consists of removable bows and a removable tarpaulin. The door header may be swivelled out. 

These two structural features greatly simplify the process of packing and unpacking the container. In particular, it is very easy to pack and unpack the container from above or through the doors by crane or crab when the roof is open and the door header is swivelled out.

It should be noted, however, that the purpose of the roof bows of an open-top container is not solely to support the tarpaulin but also to contribute to container stability. Flatracks are therefore more suitable for overheight cargoes.

Lashing rings, to which the cargo may be secured, are installed in the upper and lower side rails and the corner posts. The lashing rings may take loads of up to 1,000 kg.

Usual open-top container dimensions are 20' and 40'.

Dimensions/weights


The following are some of the most important details relating to open-top container types. The data was taken from Hapag-Lloyd, Hamburg.

| Open-top container of steel: 20' long and 8'6" high with corrugated walls, removable tarpaulin and wooden floor |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height (middle) [mm] | Height (side) [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |  |
| 5888 | 2345 | 2365 | 2315 | 30480 | 2250 | 28230 | 32.0 |
| 5897 | 2350 | 2377 | 2347 | 30480 | 2350 | 28130 | 32.5 |

| Open-top container of steel: 40' long and 8'6" high with corrugated walls, removable tarpaulin and wooden floor |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height (middle) [mm] | Height (side) [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |  |
| 12029 | 2342 | 2376 | 2326 | 30480 | 3810 | 26670 | 65.5 |
| 12022 | 2345 | 2365 | 2315 | 30480 | 3740 | 26740 | 65.3 |
| 12030 | 2350 | 2377 | 2347 | 30480 | 3850 | 26630 | 66.4 |

Dimensions of roof and door openings

The meaning of the individual letters is clear from the following Figures:

| Figure 3 | Figure 4 |
| --- | --- |

| 20' open-top container |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Roof openings |  |  |  |  | Door openings |  |  |  |  |  |
| Length | Length | Width | Width | Width | Width | Width | Width | Height | Height |  |
| A [mm] | B [mm] | C [mm] | D [mm] | E [mm] | F [mm] | G [mm] | H [mm] | I [mm] | K [mm] |  |
| 5415 | 5360 | 2205 | - | 1880 | 2335 | 1880 | 2205 | 2280 | 2125 |  |
| 5439 | 5338 | 2230 | - | 19902 | 2338 | 1902 | 2230 | 2280 | 2231 |  |

| 40' open-top container |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Roof openings |  |  |  |  | Door openings |  |  |  |  |  |
| Length | Length | Width | Width | Width | Width | Width | Width | Height | Height |  |
| A [mm] | B [mm] | C [mm] | D [mm] | E [mm] | F [mm] | G [mm] | H [mm] | I [mm] | K [mm] |  |
| 11544 | 11444 | 2230 | - | 1885 | 2336 | 1885 | 2230 | 2280 | 2146 |  |
| 11550 | 11515 | 2205 | - | 1880 | 2335 | 1880 | 2205 | 2280 | 2125 |  |
| 11573 | 11472 | 2210 | - | 1902 | 2338 | 1902 | 2210 | 2292 | 2131 |  |

Use


Open-top containers are used for all types of general cargo (dry cargo). Their principal uses are as follows:

|  | packing and unpacking from above or through the doors by crane or crab |
| --- | --- |
|  | tall cargo |

Flatracks

Description

Flatracks consist of a floor structure with a high loading capacity composed of a steel frame and a softwood floor and two end walls, which may either be fixed or collapsible. The end walls are stable enough to allow cargo securing means to be attached and several flatracks to be stacked on top of one another. Flatracks are available in 20' and 40' sizes.

A number of lashing rings, to which the cargo may be secured, are installed in the side rails, the corner posts and the floor. The lashing rings may take loads of up to 2000 kg in the case of 20' flatracks or up to 4000 kg in the case of 40' flatracks.

Some types of 20' flatracks have forklift pockets. 

40' flatracks have gooseneck tunnels at each end. In addition, they are sometimes equipped with lashing winches with 2 metric ton lashing belts.

For transport of certain cargoes, flatracks may be provided with stanchions.

Dimensions/weights

The following are some of the most important details relating to flatracks. The data was taken from Hapag-Lloyd, Hamburg [68].

| Flatrack: steel frame with fixed end walls and softwood floor, 20' long and 8'6" high |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  |  |  | Weights |  |  |
| Floor length [mm] | Length between corner posts [mm] | Floor width [mm] | Width between stanchions [mm] | Height [mm] | Height of floor [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |
| 5980 | 5698 | 2230 | 2245 | 2255 | 336 | 24000 | 2500 | 21500 |
| 5962 | 5672 | 2242 | 2242 | 2261 | 330 | 30000 | 2200 | 27800 |

| Flatrack: steel frame with collapsible end walls and softwood floor, 20' long and 8'6" high |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  |  |  | Weights |  |  |
| Floor length [mm] | Length between corner posts [mm] | Floor width [mm] | Width between stanchions [mm] | Height [mm] | Height of floor [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |
| 5950 | 5675 | 2428 | 2213 | 2270 | 316 | 33000 | 2600 | 30150 |

| Flatrack/Platform: steel frame with flushfolding end walls and softwood floor, 20' long and 8'6" high |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  |  |  | Weights |  |  |
| Floor length [mm] | Length between corner posts [mm] | Floor width [mm] | Width between stanchions [mm] | Height [mm] | Height of floor [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |
| 6038 | 5638 | 2208 | 2438 | 2235 | 370 | 30480 | 2520 | 27960 |
| 6038 | 5612 | 2210 | 2438 | 2213 | 370 | 34000 | 2740 | 31260 |

| Flatrack: steel frame with fixed end walls and softwood floor, 40' long and 8'6" high |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  |  |  | Weights |  |  |
| Floor length [mm] | Length between corner posts [mm] | Floor width [mm] | Width between stanchions [mm] | Height [mm] | Height of floor [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |
| 12010 | 11832 | 2228 | 2228 | 1981 | 610 | 45000 | 4200 | 40800 |
| 12086 | 11826 | 2224 | 2224 | 1981 | 610 | 45000 | 4200 | 40800 |
| 12010 | 11826 | 2244 | 2204 | 1981 | 610 | 45000 | 4200 | 40800 |

| High-cube flatrack: steel frame with collapsible flushfolding end walls, 40' long and 9'6" high; can be converted to a platform |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  |  |  | Weights |  |  |
| Floor length [mm] | Length between corner posts [mm] | Floor width [mm] | Width between stanchions [mm] | Height [mm] | Height of floor [mm] | Maximum weight [kg] | Tare weight [kg] | Max. payload [kg] |
| 12060 | 11660 | 2365 | 2200 | 2245 | 648 | 45000 | 5700 | 39300 |
| 12060 | 11660 | 2365 | 2200 | 2245 | 648 | 45000 | 5950 | 39050 |

The maximum payload may be used only if the load is distributed evenly over the floor structure. However, if the weight of the cargo is applied to only a small proportion of the floor, it must be distributed and the manufacturer of the flatracks may have to be consulted on safety issues.

Use

Flatracks are mainly used to transport heavy-lifts and overheight or overwidth cargoes.

Platforms (Plats)

Description

Platforms consist solely of a floor structure with extremely high loading capacity; they have no side or end walls. This high loading capacity makes it possible to concentrate heavy weights on small areas. A platform consists of a steel frame and a wooden floor structure. 

Platforms are available in 20' and 40' sizes. 40' platforms have a gooseneck tunnel at each end.

Lashing rings, to which the cargo may be secured, are installed in the side rails. The lashing rings may take loads of up to 3.000 kg.

Dimensions/weights


The following are some of the most important details relating to 20' and 40' platforms. The data was taken from Hapag-Lloyd, Hamburg.

| Platform: steel frame with softwood floor, 20' long and 1' 1 1/4" high |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Dimensions |  |  | Weights |  |  |
| Length [mm] | Width [mm] | Floor height [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |
| 6058 | 2438 | 370 | 30480 | 2520 | 27960 |
| 6058 | 2438 | 370 | 34000 | 2740 | 31260 |

| Platform: steel frame with softwood floor, 40' long and 2' high |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Dimensions |  |  | Weights |  |  |
| Length [mm] | Width [mm] | Floor height [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |
| 12192 | 2245 | 648 | 45000 | 5700 | 39300 |

Use
Platforms are used principally for oversized and very heavy cargoes.

Ventilated containers

Description

Ventilated containers are also known as passive (naturally) ventilated or coffee containers. Ventilation is provided by ventilation openings in the top and bottom side rails. The openings do not let in spray, to prevent depreciation of the cargo by rain or spray, for example.

If actively ventilated containers are required, i.e. containers with adjustable ventilation, "porthole" containers may be used, which simultaneously act as insulated or refrigerated containers. For more detailed information, see under Insulated and refrigerated containers.

Lashing rings, to which the cargo may be secured, are installed in the upper and lower side rails and the corner posts. The lashing rings may take loads of up to 1,000 kg. The common size for ventilated containers is 20'.

Dimensions/weights


The following are some of the most important details relating to ventilated containers. The data was taken from Hapag-Lloyd, Hamburg.

| Ventilated container of steel: 20' long and 8'6" high with corrugated walls and wooden floor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  | Door openings |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height [mm] | Width [mm] | Height [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |  |
| 5888 | 2325 | 2392 | 2334 | 2290 | 30480 | 2400 | 28080 | 33.0 |
| 5895 | 2321 | 2392 | 2340 | 2292 | 30480 | 2490 | 27990 | 33.0 |

Use

Ventilated containers are used especially for cargoes which have to be ventilated in transit. One of the most significant of such commodities is green coffee beans, hence the name coffee container.

Refrigerated and insulated containers

Description

Refrigerated and insulated containers are mainly available as 20' and 40' containers. A distinction may be drawn between two different systems:

1. Integral Unit (Integral Reefer Container, Integrated Unit): 

This type of refrigerated container has an integral refrigeration unit for controlling the temperature inside the container. The refrigeration unit is arranged in such a way that the external dimensions of the container meet ISO standards and thus fit into the container ship cell guides, for example. The presence of an integral refrigeration unit entails a loss of internal volume and payload.

When being transported by ship, integral units have to be connected to the on-board power supply system. The number of refrigerated containers which may be connected depends on the capacity of the ship's power supply system. If the aforesaid capacity is too low for the refrigerated containers to be transported, "power packs" may be used, which are equipped with relatively large diesel generators and satisfy ISO requirements with regard to the dimensions of a 20' container. When at the terminal, the containers are connected to the terminal's power supply system. For transport by road and rail, most integral unit refrigeration units are operated by a generator set (genset). This may either be a component of the refrigeration unit or connected to the refrigeration unit.

Air flows through the container from bottom to top. In general, the "warm" air is drawn off from the inside of the container, cooled in the refrigeration unit and then blown back in the container as cold air.

To ensure adequate circulation of the cold air, the floor is provided with gratings. Pallets form an additional space between container floor and cargo, so also forming a satisfactory air flow channel. In addition, the side walls of the container are "corrugated", which ensures satisfactory air flow there too.

In the upper area of the container, adequate space (at least 12 cm) must likewise be provided for air flow. For this purpose, during packing of the container adequate free space must be left above the cargo. The maximum load height is marked on the side walls.

To ensure vertical air flow from bottom to top, packaging must also be appropriately designed and the cargo must be sensibly stowed. 

In addition to temperature regulation, integral units also allow a controlled fresh air exchange, for example for the removal of metabolic products such as CO2 and ethylene in the case of the transport of fruits.

In the refrigeration units, both the supply and return air temperatures are measured and, depending on the operating mode, one of these values is used to control the cold air. Temperature measurement may be performed in various ways. The Partlow recorder generally records return air temperature, since this provides an indication of the state or temperature of the cargo. Data loggers are increasingly used, which detect temperature digitally and indicate it on a display. Once transferred to a PC, the data may then be evaluated.

The temperature display is attached to the outside of the refrigeration unit, so that operation of the unit may be checked at any time.

Digital or analog recorders may also be positioned directly in the cargo, so as to measure temperatures inside the container. The recorder should be accommodated in such a way that it records the temperatures at risk points in the container (inside the packaging, top layer at door end).

Integral units may be stowed both above and below deck on a ship. Above deck stowage has the advantage that the heat from return air may be more readily dissipated. However, the containers are often exposed to strong solar radiation, leading to increased refrigeration capacity requirements.

2. Porthole containers: 

This type of container is often referred to not as a refrigerated container but as an insulated container, as it has no integral refrigeration unit. The lack of a refrigeration unit allows such containers to have a larger internal volume and payload than integral units. On board, the inside of the container is supplied with cold air via the ship's central cooling plant. The air flows through the container in the same way as in integral units. Cold air is blown in at the bottom and the "warm" air is removed at the top.

Off the ship, the temperature is controlled by a terminal refrigeration system or "clip-on units". After completion of transport, the "clip-on units" may be returned using special frameworks, dimensions of which match those of a 20' container.

On the opposite end wall from the door, the containers are provided with openings for supply and return air. In general, supply air is blown into the lower opening, distributed by means of the gratings in the container floor, conveyed upwards through the cargo and discharged via the return air opening. This type of container also requires adequate air flow. For this purpose, appropriate air ducts must be provided in the floor and the ceiling and the cargo must be sensibly packaged and stowed.

Porthole containers do not have an integral temperature display. Either such a display is installed in the terminal refrigeration systems or the "clip-on units" or the temperature values may be obtained from the ship's central cooling plant.

If the porthole-containers are provided with "clip-on units" when ashore, they no longer fulfill ISO requirements with regard to dimensions.

3. General:

The doors constitute a weak point in both integral units and porthole containers. Wear to rubber door gaskets or improper handling may result in the doors no longer closing correctly, so that they are no longer sealed against rainwater and the like. During transport of chilled goods and frozen goods, water ingress may lead to cargo spoilage or to ice formation in the door area. In addition, refrigeration capacity has to be increased to compensate for losses due to cold air leakage.

In the case of frozen cargo and cargo containing non-respiring goods (goods other than fruit and vegetables), the goods are usually packed using the block stowage method. The cold air only flows around the goods and does not circulate between the boxes. Here it is important for the cargo to be pre-chilled to the required temperature before it is loaded into the container. If a load which is too warm is loaded into a refrigerated container, the heat is passed to the air and the cooling effect of the refrigeration unit is not passed to the cargo. If the air cannot pass the available cooling capacity to the cargo, it is cooled rapidly by the high cooling capacity of the refrigeration unit, and the actual cargo requires a considerably longer period for refrigeration.

Two examples of how not to do it:
A consignment of frozen goods is to be transported from Izmir to East Asia with transshipment in Egypt. Required temperature = -18°C. The cargo is too warm. The refrigerated container is not able to cool the cargo by more than 13°C within 15 days (see Figure 21).

The same consignment:
The temperature chart (see Figure 22) of a further container shows that at -10°C this cargo was too warm when it was loaded into the container. After 12 days, the temperature even rose by one degree. The daily variation of the external air temperature can be seen clearly. The reason: The supply air opening was not completely closed. Warm external air was drawn into the reefer. This was warmer during the day than by night. Despite automatic, 3-hour defrosting phases, the refrigeration unit starts to ice up. After the supply air openings are closed and an additional manual defrosting operation has been carried out, the temperature is stabilized and automatic defrosting only occurs every 12 hours. The required temperature is reached after 19 days.

Respiring goods (e.g. fruit, vegetables, plants) require the supply of a certain amount of fresh air and cooling air, depending on their metabolic activity. This restricts metabolic processes and draws off the gases produced such as ethylene and carbon dioxide. Suitable packaging such as crates, perforated plastic containers or perforated boxes must be used to allow the mixture of cooling air and fresh air to penetrate directly to the goods. Fresh air is supplied through the fresh air flaps. To allow the supply air to circulate through the cargo from bottom to top, it is necessary for the perforations in the packaging to be aligned. If the cargo is loaded on pallets, steps must be taken to ensure that the containers are arranged in such a way that the circulation of supply air is not interrupted by the base of the pallet. Steps should also be taken to avoid spaces on the floor to prevent the supply air from taking the path of least resistance (circulation bypass), thus threatening correct cooling of the goods in some areas. Circulation bypass can also be caused by slippage of the load, which means that any spaces should be filled to prevent the load from slipping. Spaces between the last row of pallets and the container door can often not be avoided. In this case, a plastic sheet can be jammed between the stack of pallets and the container door. This returns the cold air under the pallets, allowing it to reach the goods.

Ultra-low temperature refrigerated containers are capable of transporting goods at a temperature of -60°C. At temperatures of -62°C, the "eutectic point" (EP) is reached Only once the EP is reached is all the water in the cells of the product completely frozen and all microbial decomposition brought to a standstill. This means that at temperatures of below -62°C it is possible to transport or store foodstuffs for an "infinite" period without loss of quality.

Dimensions/weights

The following are some of the most important details relating to refrigerated container types. The data was taken from Hapag-Lloyd, Hamburg.

| Insulated container: 20' long and 8' high, with steel frame, walls of sandwich construction |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  | Door openings |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Max. load height [mm] | Width [mm] | Height [mm] | Gross [kg] | Tare [kg] | Net [kg] |  |
| 5724 | 2286 | 2014 | 2286 | 2067 | 24000 | 2550 | 21450 | 26,4 |
| 5770 | 2260 | 2110 | 2260 | 2090 | 24000 | 2900 | 21100 | 27,5 |
| 5770 | 2260 | 2110 | 2260 | 2090 | 27000 | 2900 | 24100 | 27,5 |

| Insulated container: 40' long and 8'6" high, with steel frame, walls of sandwich construction |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  | Door openings |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Max. load height [mm] | Width [mm] | Height [mm] | Gross [kg] | Tare [kg] | Net [kg] |  |
| 11840 | 2286 | 2120 | 2286 | 2195 | 30480 | 3850 | 26630 | 60,6 |
| 11810 | 2286 | 2210 | 2286 | 2300 | 30480 | 3650 | 26830 | 59,8 |

| Integral Unit: 20' long and 8'6" high, with steel frame, walls of sandwich construction |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  | Door openings |  | Weights |  |  | Volume [m³] | Footnote |
| Length [mm] | Width [mm] | Height [mm] | Max. load height [mm] | Width [mm] | Height [mm] | Gross [kg] | Tare [kg] | Net [kg] |  |  |
| 5479 | 2286 | 2257 | 2157 | 2286 | 2220 | 30480 | 3160 | 27320 | 28,3 | 1) |
| 5459 | 2295 | 2268 | 2168 | 2291 | 2259 | 30480 | 3050 | 27430 | 28,4 | 2) |
| 5448 | 2290 | 2264 | 2164 | 2286 | 2260 | 30480 | 3060 | 27420 | 28,3 | 2) |
| 5534 | 2316 | 2331 | 2231 | 2316 | 2290 | 30480 | 3030 | 27450 | 29,9 | 2) |
| 5529 | 2316 | 2331 | 2290 | 2316 | 2290 | 30480 | 2960 | 27520 | 29,9 | 2) |
| 5535 | 2284 | 2270 | 2224 | 2290 | 2264 | 30480 | 2942 | 27538 | 28,7 | 2) |

1) Not suitable for transporting foodstuffs
2) Suitable for clip-on generators

| Integral Unit: 40' long and 8'6" high, with steel frame, walls of sandwich construction, not suitable for transporting foodstuffs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  | Door openings |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height [mm] | Max. load height [mm] | Width [mm] | Height [mm] | Gross [kg] | Tare [kg] | Net [kg] |  |
| 11563 | 2294 | 2261 | 2161 | 2288 | 2188 | 34000 | 4600 | 29400 | 60,0 |

| Integral Unit: 40' long and 9'6" high, with steel frame, walls of sandwich construction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal dimensions |  |  |  | Door openings |  | Weights |  |  | Volume [m³] |
| Length [mm] | Width [mm] | Height [mm] | Max. load height [mm] | Width [mm] | Height [mm] | Gross [kg] | Tare [kg] | Net [kg] |  |
| 11643 | 2288 | 2498 | 2378 | 2288 | 2517 | 30480 | 4180 | 26300 | 66,5 |
| 11575 | 2294 | 2560 | 2440 | 2286 | 2570 | 32500 | 4300 | 28200 | 68,0 |
| 11568 | 2290 | 2509 | 2389 | 2290 | 2473 | 32480 | 4240 | 28240 | 66,4 |
| 11580 | 2288 | 2498 | 2378 | 2288 | 2517 | 30480 | 4180 | 26300 | 66,2 |
| 11580 | 2290 | 2513 | 2393 | 2290 | 2522 | 30480 | 4180 | 26300 | 67,0 |
| 11580 | 2286 | 2528 | 2408 | 2286 | 2545 | 30480 | 4000 | 26480 | 67,0 |
| 11580 | 2286 | 2515 | 2395 | 2286 | 2535 | 30480 | 4150 | 26330 | 67,0 |
| 11578 | 2295 | 2550 | 2425 | 2290 | 2560 | 30480 | 4640 | 25840 | 67,8 |
| 11585 | 2290 | 2525 | 2405 | 2290 | 2490 | 34000 | 4190 | 29810 | 67,0 |
| 11577 | 2286 | 2525 | 2400 | 2286 | 2490 | 34000 | 4110 | 28890 | 66,8 |
| 11577 | 2286 | 2532 | 2407 | 2294 | 2550 | 34000 | 4190 | 29810 | 67,0 |
| 11583 | 2286 | 2532 | 2412 | 2294 | 2550 | 34000 | 4120 | 29880 | 67,0 |
| 11595 | 2296 | 2542 | 2402 | 2294 | 2550 | 34000 | 4190 | 29810 | 67,7 |
| 11578 | 2280 | 2525 | 2400 | 2276 | 2471 | 34000 | 4150 | 29850 | 66,8 |
| 11578 | 2280 | 2525 | 2400 | 2276 | 2471 | 34000 | 4240 | 29760 | 66,8 |
| 11578 | 2296 | 2542 | 2402 | 2294 | 2550 | 34000 | 4300 | 29700 | 66,7 |

Use


Refrigerated containers are used for goods which need to be transported at a constant temperature above or below freezing point. These goods are divided into chilled goods and frozen goods, depending on the specified transport temperature. They principally include fruit, vegetables, meat and dairy products, such as butter and cheese. 

High-cube integral units are used in particular for voluminous and light goods (e.g. fruit, flowers).

Nowadays, goods requiring refrigeration are mostly transported in integral units, which have a markedly higher market share than porthole containers.

Chilled meat is sometimes also transported hanging, for which purpose the ceilings of refrigerated containers are equipped with special hook rails.

Special controlled atmosphere refrigerated containers are available for transporting fruit and vegetables which may be stored for a longer period in a controlled or modified atmosphere.

The atmosphere is usually established by flushing the container with nitrogen and CO2. During transport, the atmosphere is regulated by nitrogen flushing or CO2 and ethylene scrubbers. Controlled atmosphere containers must be as gastight as possible to prevent ambient air (oxygen) from penetrating.

A number of manufacturers supply the refrigerated container market with controlled atmosphere systems which may be installed in integral refrigerated containers. Controlled atmosphere systems for porthole containers are also available. In recent years, the large refrigeration unit manufacturers have acquired an increasing share of the market for standalone controlled atmosphere containers.

Bulk containers

Description

Bulk (or bulk cargo) containers have three loading hatches in the roof, each of a diameter of approx. 455 mm (1 3/4'). The distance between the hatches (center to center) is 1.83 m (6'). On the door side, there are two discharge hatches, which are sometimes equipped with short discharge tubes for guiding the bulk cargo. Alternatively, two unloading hatches may be mounted in the doorways, for emptying the containers.

Such containers may also be used for general cargo. Lashing rings are mounted in the top side rails for securing the cargo. Some bulk containers are equipped with forklift pockets, which allow handling by forklift trucks.

Dimensions/weights

The following are some of the most important details relating to bulk containers (source: Hapag-Lloyd, Hamburg.

| Internal dimensions |  |  | Door openings |  | Weights |  |  | Volume [m³] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length [mm] | Width [mm] | Height [mm] | Width [mm] | Height [mm] | Max. gross wt. [kg] | Tare wt. [kg] | Max. payload [kg] |  |
| 5934 | 2358 | 2340 | 2335 | 2292 | 24000 | 2450 | 21550 | 32.9 |
| 5931 | 2358 | 2326 | 2335 | 2292 | 24000 | 2370 | 21630 | 32.9 |

Use


Bulk containers are used in particular for transporting bulk cargo, such as grain, feedstuffs, spices. However, they may also be used for transporting general cargo.

Tank containers

Description


Tank containers must be at least 80% full, to prevent dangerous surging of the liquids in transit. On the other hand, they must not as a rule be over 95% full, or there will not be sufficient ullage space for thermal expansion. The extent of thermal expansion may be calculated for each cargo on the basis of the following formula:

ΔV = Va · γ · ΔT

Ve = Va (1 + γ · ΔT)

| ΔV | : | change in volume |
| --- | --- | --- |
| Va | : | volume at initial temperature a |
| Ve | : | final volume at temperature e |
| γ | : | coefficient of cubic (thermal) expansion |
| ΔT | : | temperature difference in degrees kelvin |

Tank containers intended for transporting foodstuffs must be labeled "Potable Liquids only".
Some hazardous materials must be transported in tank containers with no in- or outlet openings below the surface of the liquid.

Tank containers are generally designed for an operating pressure of up to 3 bar (above atmospheric). The test pressure used is 4.5 bar (above atmospheric).

If the cargo requires temperature-controlled transport, tank containers can be equipped with insulation or heating. The temperature of the cargo may be precisely controlled using temperature sensors.

Dimensions/weights

The following are some of the most important details relating to tank containers.

| 20' tank container |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| External dimensions |  |  | Weights |  |  |
| Length External dimension to ISO [mm] | Width External dimension to ISO [mm] | Height External dimension to ISO [mm] | Max. gross wt. [kg] | Tare weight [kg] | Max. payload [kg] |
| 6058 | 2438 | 2438 | 30480 | 4190 | 26290 |
| 6058 | 2438 | 2591 | 30480 | 4190 | 26290 |

Use

Tank containers are used for liquid cargoes, such as:

|  | Foodstuffs: fruit juices, spirits, sweet oils |
| --- | --- |
|  | Chemicals: hazardous materials, such as fuels, toxic substances, corrosion protection agents |